Network Testing and Validation Guide

This document provides the step-by-step procedures to verify full functionality of the network.  

 1. VLAN and Trunk Verification 
Run the following commands on every switch to verify VLANs and trunks:

- Business Building Main Switch

  - Show vlan brief and Show interfaces trunk

- Main Campus Dist Switch

  - Show vlan brief and Show interfaces trunk

2. DHCP Verification

Ensure each VLAN is receiving DHCP addresses from the Main Campus Dist switch.

Expected DHCP Ranges:

3. Inter-VLAN Routing Test

From any PC (for example, an Admin PC), test connectivity between VLANs:

4. End-to-End Device Testing

Student PC 1 Admin PC 1

Confirm all devices can reach each other. From Admin PC, ping:

ping 10.0.30.11   (Student Laptop)

ping 10.0.40.12   (Guest Laptop)

ping 10.0.50.10   (Email Server) 
ping 10.0.50.20   (Authentication Server)

From Student Laptop, ping: 
ping 10.0.10.11   (Admin PC)

ping 10.0.20.11   (Faculty PC)

5. Server VLAN (50) Test

From Faculty PC, verify communication with both servers: 
ping 10.0.50.22   (Email Server)

ping 10.0.50.21   (Authentication Server)

From each server, confirm it can reach all gateways: 
ping 10.0.10.1

ping 10.0.20.1 
ping 10.0.30.1 
ping 10.0.40.1 
ping 10.0.50.1

6. Internet Simulation (Optional)

If routers and Cloud are configured, confirm external routing: 
ping 8.8.8.8

If unsuccessful, check static routes using 'show ip route' and correct next-hop addresses on each router.

7. Save and Document

Once testing passes, save configurations and take screenshots for your report:

wr mem

Required Screenshots: 
- show vlan brief 
- show interfaces trunk 
- show ip dhcp binding 
- Ping tests between VLANs and servers 
- DHCP confirmation on PCs and laptops

Save the Packet Tracer file as 'Project1_Final_Network.pkt'.