## 🧭 1. Finalize Network Implementation in Packet Tracer

Your topology is ready — now you need to configure it to match your description and demonstrate a fully functional, secure, scalable network.

### 🔸 a. VLAN & IP Addressing

- Create VLANs for:

  - Admin

  - Faculty

  - Students

  - Guests

  - Servers

- Assign unique subnets to each VLAN (e.g., 10.0.10.0/24 for Admin, 10.0.20.0/24 for Faculty, etc.).

- Configure SVIs on the Layer 3 switches or router subinterfaces for inter-VLAN routing.

- Assign PCs to the correct VLANs and IP pools (static or DHCP).

👉 Why: This shows scalability and hierarchical design in practice.

### 🔸 b. Routing

Choose either:

- OSPF across the core routers and Layer 3 switches → recommended for a 3-person team.

- Or static routes for a simpler but less dynamic setup.

- Configure routing between VLAN subnets.

- Add a default route to the internet cloud via Router1 or Router2.

- Ensure redundancy between Router1 and Router2 (e.g., two equal-cost OSPF paths).

👉 Why: Demonstrates interconnectivity and redundancy between major areas.

### 🔸 c. DHCP, DNS, and Server Configurations

- Configure DHCP pools on routers or servers for each VLAN (except servers, which should use static IPs).

- Configure DNS and Web/Email Server services (just basic Packet Tracer services are fine).

- Configure the Authentication Server (local or RADIUS simulation) for SSH or Wi-Fi authentication.

👉 Why: Shows performance, usability, and centralized services.

### 🔸 d. Security Features

Implement the security measures listed in your description:

- ACLs at the distribution or router level:

  - Deny Student VLAN → Admin VLAN

  - Allow Faculty VLAN → Servers

  - Allow Guests → Internet only

- Port Security on Admin/Faculty switches (limit MAC per port).

- DHCP Snooping on switches.

- SSH and strong passwords on routers/switches (no telnet).

- Firewall-like ACLs on Router1 or Router2 to protect internal networks from the Cloud.

👉 Why: This is where you prove “secure network” isn’t just in the title.

### 🔸 e. Testing & Validation

Systematically test:

- Ping between VLANs (where allowed)

- Ping denied (e.g., student → admin)

- SSH into routers/switches from Admin PC

- DHCP assignment across VLANs

- Server reachability (Faculty/Admin can access, Guest cannot except internet)

👉 Why: These tests become your demo content for the video presentation.

## 📝 2. Write the Final Report (≥ 3 pages)

Structure it like this (this matches grading rubrics almost everywhere):

### Page 1: Project Overview

- Title, Team Members, Course

- Topic, Motivation, Major Areas (basically your checkpoint 1 content)

### Page 2: Design & Implementation

- Network Topology Diagram (your screenshot)

- VLAN/IP Address Table

- Routing Plan (OSPF/static, how routers are connected)

- Security Features Summary Table

### Page 3+: Testing & Workload

- Testing Table (e.g., “Faculty → Server: Ping ✅ / Student → Admin: Ping ❌”)

- Workload Distribution Table (e.g., Juan = Core/Routing, Salissa = Access/VLAN, Noah = Security)

- Challenges / Lessons Learned (short paragraph)

- Link to your presentation video (public Google Drive or YouTube)

👉 Why: The report shows planning, implementation, and individual contributions clearly — and it’s required.

## 🎥 3. Prepare the 10-Minute Presentation Video

This is your big deliverable. Structure it like this to match instructions:

### 🧍‍♂️ Each team member must speak

Assign sections to each person (e.g., Juan = Core/Intro, Salissa = VLAN/Access, Noah = Security/Demo).

### 📝 Suggested Flow

- Introduction (Team intro, project title, what scenario you picked)

- Motivation (Why school network? Security, scalability, realistic use case)

- Methodology

  - Topology design process

  - VLAN and IP planning

  - Routing decisions (OSPF/static)

  - Security measures

- Demonstration

  - Show working pings across VLANs

  - Show blocked traffic due to ACLs

  - Show SSH login

  - Show DHCP assignment on a PC

  - Show server access where appropriate

- Summary & Conclusion

  - Lessons learned, what worked well, future improvements

👉 Tips:

- Keep each speaker to 2–3 minutes max.

- Record in Packet Tracer and use a screen recorder (e.g., OBS, Zoom, PowerPoint screen recording).

- Upload to YouTube or Google Drive and make it public or “Anyone with the link can view.”

## 👥 4. Workload Distribution

Include in your report something like:

| Member | Responsibilities |
| --- | --- |
| Juan | Core routers, OSPF routing, Internet connection |
| Salissa | VLAN setup, IP addressing, DHCP/DNS servers |
| Noah | Security configuration (ACLs, SSH, firewall), Testing & Demo |

👉 This makes grading fair and shows everyone contributed.

## ✅ Checklist to Fully Meet Requirements

| Requirement | Status |
| --- | --- |
| Cisco Packet Tracer used | ✅ Already done |
| Secure network design | 🟡 Topology done → configure VLANs, ACLs, routing |
| Group of 3 with extra features | 🟡 Add OSPF, redundancy, ACLs |
| 3+ page report | 🔴 To write after implementation |
| Workload distribution | 🔴 Add table to report |
| 10-minute video presentation | 🔴 To record after testing |
| Each member presents their work | 🔴 Plan during video prep |
| Publicly accessible video link | 🔴 Include in final PDF |

### 🟢 In short:

You’ve finished the hardest conceptual part already (topology + design).
 Now you just need to:

- Configure VLANs, routing, DHCP, ACLs, and security features

- Test and document results

- Write your final 3+ page report

- Record a clean 10-minute team presentation

This will fully meet and likely exceed the project grading criteria.