### Slide 1: Title Slide

[Speaker: Salissa]

"Welcome, my name is Salissa Hernandez, and I'm joined by my team members, Juan Carlos Dominguez and Noah Nyabadza. Today we will be walking you through a 'Secure Multi-Tier Network Design for a School Campus', a secure and scalable network we designed and simulated.

### Slide 2-3: Introduction

[Speaker: Salissa]

"To start, let's go over the project's main objectives. Our task was to design a secure and scalable network for a small school campus. We chose a Core-Distribution-Access model, which is a standard hierarchical design. Our primary goals were to ensure reliable connectivity for different user groups, implement layered security, and build in scalability and redundancy. We used Cisco Packet Tracer to build and simulate this entire network."

### Slide 4-5: Motivation

[Speaker: Salissa]

"So, why did we choose this project? A school environment is a perfect real-world scenario because educational institutions require secure and robust networks to support administrative operations, faculty and student access, and guest Wifi. This environment has multiple user groups with different access needs. This project provided a realistic application of key concepts like hierarchical design, VLAN segmentation, access control, and redundancy.

### Slide 6-7: Network Topology

[Speaker: Salissa]

“Here’s a high-level look at our network topology. At the Core Layer, we have two routers and a Layer 3 switch handling routing and redundancy. The Distribution Layer includes two Layer 3 switches that manage VLAN routing and aggregate departmental traffic. Our Access Layer has four Layer 2 switches, one each for Admin, Faculty, Student, and Guest devices. We’ve also included two wireless access points for Student and Guest Wi-Fi, and our Email and Authentication servers operate securely on VLAN 50.

### Slide 8-9: VLAN & IP Design

[Speaker: Juan]

"To organize and secure the network, we created several VLANs. These include VLANs for Admin (10.0.10.0/24), Faculty (10.0.20.0/24), Students (10.0.30.0/24), Guests (10.0.40.0/24), and Servers (10.0.50.0/24). To ensure stability, we assigned static IPs for Admin and Servers. Faculty, Students, and Guests receive an IP address automatically via DHCP. Routing between these VLANs is handled by SVIs on our distribution switches."

### Slide 10-11: Routing & Services

[Speaker: Juan]

"For routing, we implemented OSPF on our core and distribution devices for dynamic routing. This setup provides redundancy through equal-cost paths. A default route to the Internet is configured through Router 1."

"We also configured DHCP pools for the Faculty, Student, and Guest VLANs. Our network hosts a Web and DNS server for internal resources and an authentication server for device and Wi-Fi access."

### Slide 12-13: Security Implementation

[Speaker: Juan]

"Security was a top priority, and we implemented it in layers. Our main line of defense is ACLs at the distribution and core layers. For example, we created rules to deny student traffic to the admin network, allow faculty access to the server VLAN, and restrict guests to Internet access only. We also implemented VLAN segmentation with DHCP snooping and port security on Admin and Faculty switches. Finally, all our network devices are managed securely using SSH with strong passwords."

### Slide 14-15: Results & Testing

[Speaker: Noah]

"To verify our design, we ran a series of tests. Ping and ACL tests verified our allowed and denied communication rules. We confirmed dynamic addressing for each VLAN via DHCP. We successfully tested secure SSH access from Admin PCs. Server access tests verified that Faculty and Admin could reach internal servers while guests were restricted. Most importantly, we tested our redundancy by shutting down a core router and verified that OSPF handled the failover correctly."

### Slide 16-17: Workload Distribution

[Speaker: Noah]

"This project was a team effort. Salissa was responsible for the VLAN setup, IP addressing, and DHCP & server configuration. I handled the security implementation, including ACLs, testing, and validation. Juan focused on core routing with OSPF and the Internet connection. Beyond our individual tasks, we all worked collaboratively on the design, documentation, and presentation."

### Slide 18-19: Summary & Conclusion

[Speaker: Salissa]

"In conclusion, we successfully designed and simulated a secure, stable campus network. Throughout this project, we applied key networking concepts including hierarchical network design, VLAN segmentation, dynamic routing with OSPF, and layered security controls. Using Cisco Packet Tracer  we demonstrate these real-world networking concepts. This project was a valuable experience in design, configuration, teamwork, and testing."

### Slide 20: Demo

[Speaker: Noah]

"And now, we'd like to move to a live demonstration in Cisco Packet Tracer to show you the network in action and prove some of the functionality we've just discussed."