# Network Testing and Validation Guide

This document provides the step-by-step procedures to verify full functionality of the network. 
Use these instructions to confirm that VLANs, trunks, DHCP, routing, and server connections are working properly.

## 1. VLAN and Trunk Verification

Run the following commands on every switch to verify VLANs and trunks:

show vlan brief
show interfaces trunk

Check that VLANs 10, 20, 30, 40, 50 are active and that all trunk ports allow 1,10,20,30,40,50.

## 2. DHCP Verification

Ensure each VLAN is receiving DHCP addresses from the Main Campus Dist switch. 
On each PC or laptop, go to Desktop → IP Configuration → DHCP and check that the IP, subnet, and gateway are correct.

Expected DHCP Ranges:
VLAN 10 (Admin)   → 10.0.10.x / Gateway 10.0.10.1
VLAN 20 (Faculty) → 10.0.20.x / Gateway 10.0.20.1
VLAN 30 (Student) → 10.0.30.x / Gateway 10.0.30.1
VLAN 40 (Guest)   → 10.0.40.x / Gateway 10.0.40.1
VLAN 50 (Servers) → 10.0.50.x / Gateway 10.0.50.1

## 3. Inter-VLAN Routing Test

From any PC (for example, an Admin PC), test connectivity between VLANs using ping:

ping 10.0.10.1
ping 10.0.20.1
ping 10.0.30.1
ping 10.0.40.1
ping 10.0.50.1

If any ping fails, run 'show ip interface brief | include Vlan' on Main Campus Dist to confirm SVIs are up/up.
If not, use 'interface vlan <number>' followed by 'no shutdown'.

## 4. End-to-End Device Testing

Confirm all devices can reach each other. From Admin PC, ping:

ping 10.0.30.11   (Student Laptop)
ping 10.0.40.12   (Guest Laptop)
ping 10.0.50.10   (Email Server)
ping 10.0.50.20   (Authentication Server)

From Student Laptop, ping:
ping 10.0.10.11   (Admin PC)
ping 10.0.20.11   (Faculty PC)

## 5. Server VLAN (50) Test

From Admin PC, verify communication with both servers:

ping 10.0.50.10   (Email Server)
ping 10.0.50.20   (Authentication Server)

From each server, confirm it can reach all gateways:
ping 10.0.10.1
ping 10.0.20.1
ping 10.0.30.1
ping 10.0.40.1
ping 10.0.50.1

## 6. Internet Simulation (Optional)

If routers and Cloud are configured, confirm external routing:
ping 8.8.8.8

If unsuccessful, check static routes using 'show ip route' and correct next-hop addresses on each router.

## 7. Save and Document

Once testing passes, save configurations and take screenshots for your report:

wr mem

Required Screenshots:
- show vlan brief
- show interfaces trunk
- show ip dhcp binding
- Ping tests between VLANs and servers
- DHCP confirmation on PCs and laptops

Save the Packet Tracer file as 'Project1_Final_Network.pkt'.