Theory questions

(1 pts) 1. What are the advantages of fiber optics over copper as a transmission medium? Is there any downside of using fiber optics over copper?

(1 pts) 2. Sometimes when a mobile user crosses the boundary from one cell to another, the current call is abruptly terminated, even though all transmitters and receivers are functioning perfectly. Why?

(1 pts) 3. The following data fragment occurs in the middle of a data stream for which the byte-stuffing algorithm described in the text is used: A B ESC C ESC FLAG FLAG D. What is the output after stuffing?

(1 pts) 4. In protocol 3 (See Figure 3-14 from the book), is it possible for the sender to start the timer when it is already running? If so, how might this occur? If not, why is it impossible?

(1 pts) 5. In protocol 6 (See Figure 3-21 from the book), when a data frame arrives, a check is made to see if the sequence number differs from the one expected and no_nak is true. If both conditions hold, a NAK is sent. Otherwise, the auxiliary timer is started. Suppose that the else clause were omitted. Would this change affect the protocol’s correctness?

(1 pts) 6. Suppose that the three-statement while loop near the end of protocol 6 (See Figure 3-21 from the book) was removed from the code. Would this affect the correctness of the protocol or just the performance? Explain your answer.

Lab questions

(1.5 pts) 7. Please read the following instructions:

- Complete the 2nd course from the bottom of this page (linkLinks to an external site.), course name: Exploring networking using cisco packet tracer.

- Open the LAN that you designed in the first assignment (Q3) on Cisco Packet Tracer and configure the devices correctly.

- You should be able to run the simulation successfully.

- You have to write a short report (1-2 paragraph) describing your network and simulation. It should include what type of physical layer, error control and data link protocol you will choose to use for your network.

(2 pts) 8. Download this file Wireshark_802_11.pcap Download Wireshark_802_11.pcap. This trace (Computer Networking: A top down approach, 8th edition) was collected using AirPcap and Wireshark running on a computer in the home network of one of the authors, consisting of a Linksys 802.11g combined access point/router, with two wired PCs and one wireless host PC attached to the access point/router. The author is fortunate to have other access points in neighboring houses available as well. Now read the following instructions:

- Open the trace file on Wireshark and look at the different protocol details collected in different time frames.

- Can you identify how many of these protocols are part of Data link layer (partially or completely)? List them.

- Please pick any 5 of them and then briefly explain each of the capture and it's purpose from your observation. Your report should include screenshot of the specific capture, a short explanation of observed similar or different patterns in terms of source, destination, packets, lengths, and other information relevant to the capture.

(1.5 pts) 9. Implement Code Division Multiplexing (CDM) in Python programming language. Your input should be the chip sequences and the transmissions, output should be recovery of each station's signal. Use the chip sequences of the stations from the example from slide/book. Print the sample input-output of that example at the end of your code.

(4 pts) 10. Implement Protocol Selective Repeat in Python programming language. Your input should be the frames (at least 7) and output should be the status (data/ack/nack - - sent/received/lost) of each data and control frame until the transmission is complete. Please include any error detection or correction algorithm as part of the error control in your protocol.

Exceptional question (7000 level students must need to answer, 1 point will be deducted if you don't)

11. Research on point-to-point protocol (PPP) and write a brief description on it. Specify which functionalities are common and which are different between PPP and protocol 6: Selective Repeat that we studied in class.

Submission Instruction:

Please write code in jupyter notebook for question no 9 and 10 and submit .html version of your code with generated sample input-output on it. The rest of the answers should be submitted in a single pdf.