Assignment 3 Instructions

Submit one assignment per group and list all of the team members name on the top

Theory questions

(1 pts) 1. A group of friends gets together to play highly interactive CPU- and network-intensive video games. The friends play together using a high-bandwidth wireless network. The wireless signal cannot propagate through walls, but the friends are all in the same room. In such a setup, would it be best to use nonpersistent CSMA or the token ring protocol? Please explain your answer.

(1 pts) 2. Consider the network of Fig. 5-12(a) in your textbook. Distance vector routing is used, and the following link state packets have just come in at router D: from A: (B: 5, E: 4); from B: (A: 4, C: 1, F: 5); from C: (B: 3, D: 4, E: 3); from E: (A: 2, C: 2, F: 2); from F: (B: 1, D: 2, E: 3). The cost of the links from D to C and F are 3 and 4 respectively. What is D’s new routing table? Give both the outgoing line to use and the cost.

(1 pts) 3. Consider the packet queues shown in Fig. 5-29 in your textbook. What is the finish time and output order of the packets if the middle queue, instead of the bottom queue, has a weight of 2? Order packets with the same finish time alphabetically. The start time is either the finish time of the previous packet, or the arrival time of the packet, if the queue is empty when it arrives.

(1 pts) 4. Looking at the network of Fig. 5-6 in your textbook, how many packets are generated by a broadcast from B, using a) reverse path forwarding? and b) the sink tree?

Lab questions

(2 pts) 5. There are different protocols in network and transport layer including TCP, UDP, IPv4, IPv6, ARP, ICMP, and many more. Please pick any 5 of them and pick a capture for each protocol at your local network using Wireshark. Then briefly explain each of the capture and relevant packets from your observation. Your report should include screenshot of your captures, relevant packets in each capture and their purposes, any pattern that you observe, and other properties you can visualize that match with the book/lecture description.

(2 pts) 6. Implement weighted fair queuing in python language. Your algorithm should follow the book and your sample input-output should match the Fig. 5-29 (a) and (b). Your output should be just the order in the round robin sessions.

(2 pts) 7. Suppose you are tasked with designing an IPv6 network for SMU, which has 8 distinct schools. Each school has multiple departments that need unique IPv6 addresses for their computers, servers, and other devices. Your job is to allocate subnets within the university's IPv6 address block, taking into account the number of departments in each school and their anticipated needs. Consider the following for your allocation.

- SMU University has been assigned the IPv6 address block 2001:0db8:abcd::/48.

- Allocate unique subnet addresses for each school, assuming that each school will have at least 4 departments, with each department needing at least 100 IP addresses (varies according to need).

- Ensure that each school's subnet allows for future expansion of departments and addresses per department.

For each school and department’s subnet allocation, provide the following information:

- The full IPv6 subnet address.

- The range of usable addresses within the subnet.

- A brief justification for your design choices, especially if a department may need additional addresses in the future.

Exceptional question (7000 level students must need to answer, 1 point will be deducted if you don't)

8. Implement one of the routing algorithms discussed in class.

Submission Instruction:

Please write code in jupyter notebook for question no 6 and 8 and submit .html version of your code with generated sample input-output on it. The rest of the answers should be submitted in a single pdf.

Keep in mind, this submission will count for everyone in your Project Groups group.